Speed Ramps Testing

Start from a certain speed, command positive acceleration and wait until speed is reached. Hold at steady state for 10 seconds then command deceleration.

| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| --- | --- | --- | --- | --- | --- |
| 0-20 mph | ±0.25 | x | 9/16 | ok | 62409002 |
|  | ±0.5 | x | 9/16 | ok | 62409002 |
|  | ±0.75 | x | 9/16 | ok | 62409002 |
|  | ±1 | x | 9/16 | ok | 62409002 |
|  | ±1.5 | x | 9/16 | ok | 62409002 |
|  | ±2 | x | 9/16 | Ok, this is the max/min acceleration at this speed | 62409002 |
|  | ±2.25 |  |  | System fault |  |
|  | ±3 |  |  |  |  |

| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| --- | --- | --- | --- | --- | --- |
| 10-30 mph | ±0.25 | x | 9/16/24 | ok | 624009009 |
|  | ±0.5 | x | 9/16/24 | ok | 624009009 |
|  | ±0.75 | x | 9/16/24 | ok | 624009009 |
|  | ±1 | x | 9/16/24 | ok | 624009009 |
|  | ±1.5 | x | 9/16/24 | ok | 624009009 |
|  | ±2 | x | 9/16/24 | ok | 624009009 |
|  | ±2.01 (1st) | x | 9/16/24 | Vehicle came down to a stop by itself | 624009009 |
|  | ±2.01 (2nd) | x | 9/16/24 | System faulted | 624009009 |
| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| 20-40 mph | ±0.25 | x | 9/16 | - Vehicle takes some time to start from 0, need to look into data to know how much - To maintain speed @ 40mph you need to send some acceleration (0.03 mps2) | 624009003 |
|  | ±0.5 | x | 9/16 | ok | 624009003 |
|  | ±0.75 | x | 9/16 | ok | 624009003 |
|  | ±1 | x | 9/16 | ok | 624009003 |
|  | ±1.5 | x | 9/16 | ok | 624009003 |
|  | ±2 | x | 9/16 | Ok, this is the max/min acceleration at this speed | 624009003 |
|  | ±2.25 |  |  | System fault |  |
|  | ±3 |  |  |  |  |

| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| --- | --- | --- | --- | --- | --- |
| 30-50 mph | ±0.25 | X | 9/16/2024 | To maintain 30 mph send 0.025 mps2 To maintain 50 mph send 0.08 mps2 | 624009010 |
|  | ±0.5 | X | 9/16/2024 | ok | 624009010 |
|  | ±0.75 | X | 9/16/2024 | ok | 624009010 |
|  | ±1 | x | 9/16/2024 | ok | 624009010 |
|  | ±1.5 | x | 9/16/2024 | ok | 624009010 |
|  | ±2 | x | 9/16/2024 | ok | 624009010 |
|  | ±2.01 | x | 9/16/2024 | System fault | 624009010 |
|  | ±3 |  |  |  |  |

| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| --- | --- | --- | --- | --- | --- |
| 40-60 mph | ±0.25 | x | 9/16/2024 | To maintain 60 mph, send 0.125 mps2 To maintain 40 mph, send 0.035 mps2 | 624009004 |
|  | ±0.5 | x | 9/16/2024 | ok | 624009004 |
|  | ±0.75 | x | 9/16/2024 | ok | 624009004 |
|  | ±1 | x | 9/16/2024 | ok | 624009004 |
|  | ±1.5 | x | 9/16/2024 | ok | 624009004 |
|  | ±2 | x | 9/16/2024 | Ok, this is the max/min acceleration at this speed | 624009004 |
|  | ±2.25 |  |  | System fault |  |
|  | ±3 |  |  |  |  |

| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| --- | --- | --- | --- | --- | --- |
| 50-70 mph | ±0.25 | x | 9/16/2024 | To maintain 50 mph send 0.08 mps2 To maintain 70 mph send 0.19 mps2 | 624009005 |
|  | ±0.5 | x | 9/16/2024 | Done twice | 624009005 |
|  | ±0.75 | x | 9/16/2024 | ok | 624009005 |
|  | ±1 | x | 9/16/2024 | ok | 624009005 |
|  | ±1.5 | x | 9/16/2024 | ok | 624009005 |
|  | ±2 | x | 9/16/2024 | Ok, this is the max/min acceleration at this speed | 624009005 |
|  | ±2.1 |  |  | System fault | 624009005 |
|  | ±3 |  |  |  | 624009005 |

Acceleration Limit Testing

Start from a certain speed, command acceleration, check system functionalities

| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| --- | --- | --- | --- | --- | --- |
| Start @0mph | ±1 | x | 9/16/2024 | ok | 624009007 |
|  | ±1.5 | x | 9/16/2024 | ok | 624009007 |
|  | ±1.8 | x | 9/16/2024 | Done twice | 624009007 |
|  | ±1.83 | x | 9/16/2024 | Ok, done twice | 624009007 |
|  | +1.84 | x | 9/16/2024 | Went up to ~55 mph then clipped off and started decelerating | 624009007 |
|  | +1.85 | x | 9/16/2024 | Went up to ~60 mph then clipped off and started decelerating | 624009007 |
|  | +1.89 | x | 9/16/2024 | Went up to ~55 mph then clipped off and started decelerating | 624009007 |
|  | +1.9 | x | 9/16/2024 | Went up to ~50 mph then clipped off and started decelerating | 624009007 |
|  | +2 | x | 9/16/2024 | Went up to ~50 mph then clipped off and started decelerating | 624009007 |
|  | ±2.1 |  |  |  |  |

| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| --- | --- | --- | --- | --- | --- |
| Start @60mph | ±1 | x | 9/16/2024 | Done twice | 624009006 |
|  | ±1.5 | x | 9/16/2024 | ok | 624009006 |
|  | ±2 | x | 9/16/2024 | Ok, this is the max/min acceleration at this speed | 624009006 |
|  | ±2.1 |  |  | System fault |  |

| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| --- | --- | --- | --- | --- | --- |
| Start @65mph | ±1 |  |  |  |  |
|  | ±1.5 |  |  |  |  |
|  | ±2 | x | 9/16/2024 | Ok, this is the max/min acceleration at this speed | 624009006 |
|  | ±2.1 |  |  | System fault |  |

Deceleration Limit Testing

Start from a certain speed, command deceleration, check system functionalities

| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| --- | --- | --- | --- | --- | --- |
| Start @20mph | -2 | X | 9/16/2024 | ok | 624009011 |
|  | -2.01 | x | 9/16/2024 | ok | 624009011 |
|  | -2.1 | x | 9/16/2024 | ok | 624009011 |
|  | -2.5 | x | 9/16/2024 | ok | 624009011 |

| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| --- | --- | --- | --- | --- | --- |
| Start @40mph | -2 | x | 9/16/2024 | ok | 624009012 |
|  | -2.5 | x | 9/16/2024 | ok | 624009012 |
|  | -3 | x | 9/16/2024 | ok | 624009012 |

| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| --- | --- | --- | --- | --- | --- |
| Start @60mph | -2 | x | 9/16/2024 | ok | 624009013 |
|  | -2.5 | x | 9/16/2024 | ok | 624009013 |
|  | -3 | x | 9/16/2024 | Ok, but tire pressure sensor and parking brake faulted | 624009013 |
|  | -3.5 |  |  |  |  |

Starting from Stop Testing

| Speed [mph] | Acceleration [mps2] | Completion Check | Date | Notes | Test # |
| --- | --- | --- | --- | --- | --- |
| 0mph | ±0.5 (no resume) | x | 9/16/24 | Time: ~3sec | 624009008 |
|  | ±0.5 (with resume pedal) | x | 9/16/24 | Kicks out of openpilot override when pedal is pressed | 624009008 |
|  | ±0.5 (resume button) | x | 9/16/24 | Done twice, time: ~2 sec | 624009008 |
|  | ±0.5 (no resume)- 2nd trial | x | 9/16/24 | Time : ~2.40 sec | 624009008 |
|  | ±0.5 (resume button) 2nd trial | x | 9/16/24 | Time : ~2 sec | 624009008 |