Hardware:

- CV2X Radios (OBU and RSU): Used for bi-directional V2X communication between the vehicle (On-Board Unit) and the infrastructure (Roadside Unit).

- Traffic Signal Controller: Interfaces with RSUs to broadcast MAP and SPaT messages conforming to SAE J2735 standards.

- Raspberry Pi: Acts as the edge processor in the vehicle, responsible for receiving, decoding, and processing V2X data in real-time.

Software:

- Objective Systems ASN.1/C Libraries (https://obj-sys.com/): Provides decoding and encoding support for SAE J2735 message sets including MAP, SPaT, and BSM. This tool enables reliable interpretation of V2X messages received over CV2X interfaces.

- Map Engine Library (https://github.com/mmitss/mmitss-common): An open-source C++ library developed under the connected vehicle pool fund study project, MMITSS that processes MAP messages and enables map-based localization and signal group identification.

Architecture Overview:

The proposed system will build upon a modular framework developed in prior initiatives such as CDA-ORNL, APCK-I, and VOICES, leveraging tools and communication protocols to support ENACTED project.

At the infrastructure level, Traffic Signal Controllers broadcast SAE J2735-compliant MAP and SPaT messages via CV2X RSUs. These messages contain geospatial intersection geometry, signal phase timing, and lane-level metadata necessary for vehicle-to-infrastructure coordination.

On the vehicle side, a test vehicle equipped with a CV2X OBU listens for these V2X messages and simultaneously monitors its own GNSS coordinates or BSM (Basic Safety Messages). These data streams are sent over the Ethernet to an onboard Raspberry Pi, which serves as the on-board V2X data processing unit.

A software module running on the Raspberry Pi performs the following key functions:

- Message Decoding:
Incoming MAP and SPaT messages are decoded using the Objective Systems ASN.1 library to extract structured data from raw binary messages.

- Map Matching and Localization:
Using the Map Engine Library, the system identifies the appropriate map based on decoded reference points and current GPS coordinates. This enables real-time localization of the vehicle within the intersection’s geometry.

- VSP (Vehicle Signal Processor) Calculations:
Once the vehicle is localized on an active map, the system determines the:

  - Current approach ID the vehicle is on

  - Lane ID and movement direction

  - Corresponding signal group associated with that lane

Following localization, signal phase and timing (SPaT) information is retrieved to understand the current and projected signal status for the vehicle’s movement.

- Integration with Speed Advisory Module (MABXII):
The identified lane, signal group, and signal phasing information will be forwarded to the MABXII which will run the Speed Advisory application.